VortexDeep

LinkedIn

26W39-2 / Engineering PID Simulator Tool

Last week I showed three tuning philosophies for the same PID loop — same math, three completely different personalities.

The obvious next question: where do those numbers actually come from?

Here's the method most tuning guides skip. Feed the process a sine wave and sweep its frequency. As frequency rises, the output shrinks and falls further behind — more lag, smaller amplitude. Keep raising it until the lag hits 180°. That single point tells you everything: how far you've swept gives you the ultimate period, how much the output has shrunk gives you the ultimate gain.

Two numbers. Every tuning rule from last week — aggressive, balanced, conservative — is just a different recipe built on the same Ku and Pu.

That's the same instinct I bring to automating a workflow at VortexDeep: find the number that actually governs the system, then build every decision on top of it — not a guess.

What's the "Ku and Pu" in your process — the one measurement that would settle an argument instead of guessing? 👇

Q: great it works no errors. many thanks, what these metrics represent?

A: Good question — worth spelling out plainly since the tool's been treating them as inputs to a formula this whole time without ever saying what they are.

Ku (ultimate gain) — the proportional gain at which the loop would oscillate forever at a constant amplitude, if you used a pure P controller with no I or D. Push the gain any lower and oscillations die out; push it any higher and they grow. Ku is the exact knife-edge between the two — the loop's natural "wobble point."

Pu (ultimate period) — how long one of those sustained oscillations takes, in seconds. It's a property of the process itself (how fast it naturally wants to cycle once pushed to that edge), not of the controller.

Why the −180° phase crossing gives you both: a feedback loop goes unstable exactly when two things happen at the same frequency — the signal going around the loop comes back exactly inverted (180° phase lag) and exactly the same size it started (loop gain = 1). A plain P controller adds no phase shift of its own, so the process alone has to supply that full 180° lag — which is precisely the frequency your sine-sweep chart found. And since the controller just scales the signal, the gain needed to make the total loop gain hit 1 at that frequency is simply 1 divided by however much the process has already shrunk the signal there. That's Ku. The period of the wave at that same frequency is Pu.

Why they matter: every tuning rule in the tool — classic ZN, Tyreus-Luyben, no-overshoot — is just a different, safer fraction of these two numbers (e.g. classic PID runs at roughly 0.6×Ku, with Ti and Td set as fractions of Pu). Nobody actually runs a loop at Ku — that's the edge of instability. Ku and Pu are the reference point every real setting backs off from, by different amounts depending on how cautious you want to be.

One practical note: the original Ziegler-Nichols method found these by actually cranking up gain on a real, running process until it started oscillating — useful, but risky on anything expensive or dangerous to push that far. What your tool does instead — sweeping the math model with a virtual sine wave — gets you the same Ku and Pu without ever touching the real process.

Q: in terms of real regulated “process” what it means? What is practical usefullnes of it?

A: Good follow-up — the math makes more sense once it's tied to something physical.

What Ku physically is

Take a real loop — say a temperature controller on a heater, or level control on a tank — and strip it down to pure proportional control (no I, no D). Now slowly turn the gain up. At low gain, if something disturbs the process, it wobbles and settles back down. As you keep raising the gain, that wobble takes longer to die out. At one specific gain, it stops dying out entirely — the process value oscillates back and forth at a constant, unchanging amplitude, forever, neither growing nor shrinking. That exact gain is Ku. Turn it up even one more notch and the oscillation starts growing — valve swinging harder and harder, temperature swinging wider and wider, genuinely dangerous territory (trips, damage, spilled product).

So Ku isn't an abstract number — it's the literal gain setting where the physical process sits right on the edge between "self-correcting" and "runs away."

What Pu physically is

While it's doing that sustained oscillation, time one full cycle — peak to peak. That's Pu, in seconds. It's not a property of the controller at all — it's how fast the process itself naturally wants to cycle once pushed to that edge. A twitchy little flow loop might have a Pu of a couple seconds; a big tank of liquid slowly heating up might have a Pu measured in minutes.

Why this is practically useful

- A real starting point instead of a guess. Every serious tuning method (classic ZN, Tyreus-Luyben, the no-overshoot variant — everything in your tool) is just "some safe fraction of Ku, some safe multiple or fraction of Pu." Without Ku/Pu, tuning is trial-and-error nudging on a live process. With them, you have numbers to plug into a formula and land close on the first try.

- A sanity check on existing tuning. If you know a loop's Ku and someone finds its gain set to, say, 80% of that — that loop is basically living on the edge of instability, and now you know exactly why it's been acting twitchy. If gain is at 2% of Ku, the loop's probably sluggish and slow to reject disturbances. Ku/Pu turn "this loop feels off" into "this loop is at 0.8×Ku, that's too hot" — a number, not a vibe.

- A diagnostic on the process itself. An unusually short Pu can point to valve stiction, noisy sensors, or equipment that's just faster/twitchier than whatever it's paired with — useful even outside tuning.

One important real-world caveat

Actually pushing a live, real process to sustained oscillation — the textbook way of finding Ku/Pu — is often too risky to do for real: you don't want a reactor or a compressor oscillating at growing amplitude to find out where it breaks. That's why two safer alternatives exist:

- Relay auto-tuning (this is in your project's reference PDFs) — deliberately induce a small, bounded on/off oscillation instead of a linear one, estimate Ku/Pu from that, without ever letting the process actually run away. This is what real industrial auto-tuners (DCS/PLC "auto-tune" buttons) generally do.

- Model-based, like your tool — if you already know the process's gain, lag, and delay (usually from a simple step test: bump the valve, watch the response curve, read off those three numbers), you can compute Ku/Pu purely mathematically, zero risk to the real process. The trade-off: it's only as good as how well gain/lag/delay actually describe the real thing.

That last point is honestly a good hook for a third post if you want it — "why you'd risk oscillating a real reactor to tune it, when you can just simulate it first" is a genuinely compelling, very on-brand VortexDeep argument (measure and simulate before you act on the real system).